# Microtile Serial Commands - 1.0.13

## Port Setup

Serial Port: 	115200 baud, no start bit, 8 data bits, 1 stop bit, no flow control

TCP/IP Port:	Port# 3002

Notes:

- Send a carriage return “/r” or new line “/n” at the end of your output, this will start the processing of command.

- The command and parameter are case sensitive. Make sure you use capital letters for the commands.

## Terminal Program Setup

To setup terminal program for proper echoing, see below:

### TeraTerm:

Setup->Terminal->Local echo: checked

Setup->Terminal->New-Line->Transmit: “CR+LF”

### HyperTerm:

File->Properties->ASCII setup-> Send line ends with line feeds: checked

File->Properties->ASCII setup-> Echo typed characters locally: checked

### TCP:

You can also use TeraTerm to connect to the TCP port of Microtiles Ecu

File->New connection->TCP->Host: Ecu’s IP address

File->New connection->TCP->Telnet: unchecked

File->New connection->TCP->TCP port#: 3002

## Commands:

## Power

### (PWR <0|1>)

Set Power State 0 – off, 1-on

### (PWR?)

Get power state, 0 – off, 1-on

## Reset

Notes: The prefix “#” and “$” has no effect on (RST+ECUS) command.

### (RST)

Reset all selected tiles and ECU(s)

### (RST+ECUS)

Reset all selected ECU(s) only

### (RST+TILS)

Reset all selected MicroTile(s)

## Video Mode

### (VDM <0|1|2>)

Set video mode on selected MicroTile(s).

0-video, 1-unscaled OSD, 2-scaled OSD

## Internal Test Pattern

### (ITP <pattern> <x> <y>)

Draw internal test pattern on selected MicroTiles(s).  MicroTiles must be set to video OSD mode to display test patterns.

- Draw OSD full field test patterns.

(ITP 0 <hex_rgb_value>)

0xFF0000 - red, 0x00FF00 - green, 0x0000FF - blue, 0xFFFFFF - white, 0x000000 - black

- Draw OSD no video signal test pattern.

(ITP 1)

- Draw OSD full field checkerboard pattern.

(ITP 2 <tile_width> <tile_height>)

- Draw OSD a color square on black background test pattern.

(ITP 3 <x> <y> <width> <height> <hex_rgb_value>)

- Draw OSD full field grid pattern.

(ITP 4 <tile_width> <tile_height>)

- Draw OSD full intensity color bars.

(ITP 5)

- Draw OSD full field inverted checkerboard pattern.

(ITP 7 <tile_width> <tile_height>)

- Draw 13-point optical pattern.

(ITP 8)

- Draw gray scale pattern.

(ITP 9)

## Network

Note:	When using the manual IP mode, please set the IP address first, the Subnet Mask second and the Gateway third.

### (NET+ETH0?)

Retrieve IP address from Ethernet port 0

### (NET+SUB0?)

Retrieve Network Subnet Mask from Ethernet port 0

### (NET+GATE?)

Retrieve default gateway from Ethernet port 0

### (NET+MAC0?)

Retrieve MAC address from Ethernet port 0

### (NET+MODE?)

Retreive the Network IP address mode: Auto or Manual

### (NET+ETH0 “x.x.x.x”)

Set IP address “x.x.x.x” to Ethernet port 0

### (NET+SUB0 “x.x.x.x”)

Set Network Subnet Mask “x.x.x.x” to Ethernet port 0

### (NET+GATE “x.x.x.x”)

Set default gateway “x.x.x.x” to Ethernet port 0

### (NET+MODE “Auto”)

Set the Network IP addressing mode to Auto (DHCP)

## List

### (LST?)

List all the MicroTiles.

### (LST+TILS?)

List all the MicroTiles.

### (LST+ECUS?)

List all the Ecus.

## Select

### (SEL?)

List all the selected Microtiles.

### (SEL+TILS?)

List all the selected Microtiles.

### (SEL+ECUS?)

List all the selected Ecus.

### (SEL <x|ALL|NONE> <y> …)

Select a number/ALL/none of Microtiles

### (SEL+TILS <x|ALL|NONE> <y> …))

Select a number/all/none of Microtiles

### (SEL+ECUS <x|ALL NONE> <y> …)

Select a number/all/none of Ecus

## Priority

### (PRI?)

Retreive the priority of the selected Ecus

### (PRI <priority>)

Set the priority of the selected Ecus

## Version

### (VER?)

Retreive the firmware and hardware version of the selected Microtiles

### (VER+TILS?)

Retreive the firmware and hardware version of the selected Microtiles

### (VER+ECUS?)

Retreive the firmware and hardware version of the selected Ecus

## Alignment

### (AGN?)

Retreive the image alignment adjustment of the selected MicroTiles.

### (AGN <top left x> <top left y> <top right x> <top right y> <bottom left x> <bottom left y> <bottom left x> <bottom left y>)

Set the image alignment adjustment of the selected Microtiles

## Display Window

### (DPW?)

Retreive the display window of global video source.

### (DPW <x offset> <y offset> <width> <height>)

Set the display window of global video source. All parameters in decimal point from 0 to 1.

## Date and time

Notes: The seconds field is not applied and is assumed to be zero

### (TMD+TIME “17:50:45”)

Sets the local time of selected Ecus

### (TMD+DATE “2007/02/30”)

Sets the local date of selected Ecus

### (TMD+TIME?)

Returns the local time from the selected Ecus:

(TMD+TIME!

Index		IP		Serial		Time

@m	0	192.168.228.45	0x0024e999	17:50:45

)

### (TMD+DATE?)

return the local date from the selected Ecus

(TMD+DATE!

Index		IP		Serial		Date

@m 	0	192.168.228.45	0x0024e999	2007/02/30

)

@ indicates the current Ecu

m indicates the master Ecu

## Mullion Reduction

### (MLR 16 4)

Sets the mullion gain to 16 and width to 4 for on all MicroTiles

### (MLR?)

Returns the current mullion gain and width of selected MicroTiles

(MLR!

Index	Serial		Gain	Width

0	0x0023ecf8	16	4

)

## Sharpness

### (SHP 50)

Sets sharpness to 50 on all MicroTiles

### (SHP?)

Returns the current sharpness of selected MicroTiles

(SHP!

Index	Serial		Sharpness

0	0x0023ecf8	50

)

## Uniformity correction

## (UCR 1)

Turns on uniformity correction on all MicroTiles

## (UCR 0)

Turns off uniformity correction on all MicroTiles

## (UCR?)

Returns uniformity correction settion of selected MicroTiles

(UCR!

Index	Serial		Uniformity Correction

0	0x0023ecf8	1

)

## Remote Control

### (RMC+PAIR)

Enables remote control pairing for 30 seconds

### (RMC+UNPR)

Unpairs all remote control

The above two commands (RMC+PAIR) (RMC+UNPR) will be available on a Master Ecu only

### (RMC+ENBL)

Enables the remote control module in an Ecu

### (RMC+DSBL)

Disables the remote control module in an Ecu

### (RMC?)

Returns the state (enabled or disabled) of the remote control module in an Ecu.

(RMC!

Index     Serial        State

ECU [0] 0x0024e89d      Enabled

)

## Color Matching

### (CCM+MODE TILE)

Sets the color match mode to tile mode.

### (CCM+MODE CANVAS)

Sets the color match mode to canvas mode.

### (CCM+MODE?)

Returns the color match mode.

(CCM+MODE! CANVAS)

### (CCM+BRTM ECON)

Sets the brightness mode to economy

### (CCM+BRTM FIXED)

Sets the brightness mode to fixed

### (CCM+BRTM MAX)

Sets the brightness mode to maximum

### (CCM+BRTM?)

Returns the brightness mode

(CCM+BRTM! MAX)

### (CCM+TBRT 800)

Sets the target brightness to 800 nits when in fixed brightness mode

### (CCM+TBRT?)

Returns the target brightness when in fixed brightness mode

(CCM+TBRT! 800)

This command is only available when in fixed brightness mode

### (CCM+ABRT?)

Returns the actual brightness

(CCM+ABRT! 700)

### (CCM+CLRM FIXED)

Sets the color mode to fixed

### (CCM+CLRM SRGB)

Sets the color mode to sRGB

### (CCM+CLRM MAX)

Sets the color mode to maximum

### (CCM+CLRM?)

Returns the color mode

(CCM+CLRM! MAX)

### (CCM+CLRT 6500)

Sets the color temperature to 6500

### (CCM+CLRT?)

Returns the color temperature

(CCM+CLRT! 6500.000000)

### (CCM+TCLR 0.64 0.33 0.3 0.6 0.15 0.06)

Sets the target color gamut to red_x=0.64 red_y=0.33 green_x=0.3 green_y=0.6 blue_x=0.15 blue_y=0.06.

This command is only available if the Color Mode is set to “FIXED”.

### (CCM+TCLR?)

Returns the target color gamut.

(CCM+TCLR! 0.64 0.33 0.3 0.6 0.15 0.06)

This command is only available if the Color Mode is set to “FIXED”

### (CCM+ACLR?)

Returns the actual color gamut achieved on the canvas.

(CCM+ACLR! 0.64 0.33 0.3 0.6 0.15 0.06)

Automatic Array Configuration.

Automatic Array Configuration refers to the ability of the Software to detect a changes in the array’s setup and configure the array. For example, addition or deletion of tiles, moving of tiles, changing of subarrays, changing of manually mapped tiles.

### (AAC 1)

Turns on automatic array reconfiguration

### (AAC 0)

Turns off automatic array reconfiguration

### (AAC?)

Returns the automatic array reconfiguration mode

(AAC!1)

## Subarray

### (SAR+LIST?)

Lists all the subarrays and their properties.

(SAR+LIST!

List of subarrays, count=1

Index array  x  y width height input enable xoffset yoffset width  height

0     0  0  0     1      1     2      1 0.00000 0.00000 1.0000 1.0000

)

### (SAR+ADD_ <array> <x> <y> <width> <height>)

Adds a subarray.

<array> - The array index.

<x> - The x coordinate of the top left MicroTiles.

<y>- The y coordinate of the top left MicroTiles.

<width> - Number of MicroTiles in the subarray in the x dimension.

<height> - Number of MicroTiles in the subarray the y dimension.

### (SAR+AUTO)

Automatically conifigure subarrays

### (SAR+DEL_ <index>)

Deletes a subarray.

<index> - the index of the subarray to delete

### (SAR+DEL_ ALL)

Deletes all the subarray.

### (SAR+ENBL <index>)

Enables a subarray.

<index> - the index of the subarray to enable.

### (SAR+DSBL <index>)

Disables a subarray.

<index> - the index of the subarray to disable.

### (SAR+DPWN <index> <x> <y> <width> <height>)

Sets the display window.

<index> - the index of the subarray

<x> - x offset in decimal point (0.0 -1.0).

<y> - y offset in decimal point (0.0 -1.0).

<width> - width of the display window in decimal point (0.0 – 1.0).

<height> - height of the display window in decimal point (0.0 – 1.0).

### (SAR+INPT <index> <type>)

Sets the input selection

<index> - index of the subarray

<type> 0 – no video, 1- global video source, 2- local video, 3 – either local or global video source

## Manual Mapping

### (MMP+LIST?)

Lists all manually mapped tiles.

(MMP+LIST!

List of manually mapped tiles, count=1

Index Serial       Array  X  Y

0     0x23cf00     0  0  0

)

### (MMP+ADD_ <index> <array> <x> <y>)

Adds a manually mapped tile

<index> - the index of the MicroTiles in the MicroTiles list

<array> - the index of the array

<x> - the x coordinate

<y> - the y coordinate

### (MMP+DEL_ <index>)

Deletes a manually mapped tile

<index> - the index of the manually mapped tile

### (MMP+DEL_ ALL)

Deletes all manually mapped tiles

## Array

### (LST+ARRY?)

Lists all the arrays

(LST+ARRY!

List of arrays, count=1

Index Id xsize ysize count

0  0     1     3     3

)

## EDID

### (EDD <hsize> <vsize> <rate>)

Writes custom edid to selected Ecus.

<hsize> - horizontal size in pixels

<vsize> - vertical size in pixels

<rate> - frame rate in Hz

### (EDD?)

Reads custom edid from selected Ecus.

(EDD!

ECU [0] H-Active: 1920 V-Active: 1080 Frame Rate:  59.93 Hz

)

## Neighbours

### (LST+NGBR?)

Lists the neighbours information.

(LST+NGBR!

I  A-X,Y    Serial     Left       Bottom     Port0        Port1        Source0

Source1

0  0- 0, 0 0x0023cf00 0xffffffff 0x0024ccc3 0x0024ccc3-0 0x0023effb-1 0x0000000

0 0x0024e89d

1  0- 0, 2 0x0023effb 0xffffffff 0xffffffff 0x0024e89d-0 0x0023cf00-1 0x0024e89

d 0x00000000

2  0- 0, 1 0x0024ccc3 0xffffffff 0x0023effb 0x0023cf00-0 0x00000000-0 0x0024e89

d 0x00000000

)

## Configure Array

### (CNG)

Configure the array - determine the location of individual tiles using the neighbourhood information and set the crop windows of all the tiles.

## Link Quality

### (LNQ?)

Request the link quality of the select MicroTiles. This is used to indicate the quality of the communication link (High Speed Serial Link) between MicroTiles.

Example:

(LNQ?)

(LNQ!

Index Serial               Ch1 Error Count           Ch2 Error Count

0 0x0023ed84                         0                         0

1 0x0023effb                        30                         0

2 0x0024ccc3                         0                         8

3 0x0024cfd9                         0                        32

)

This indicates tile 1 has 30 errors on Ch1 (HSSL Port 1) and tile 3 has 32 errors on Ch2 (HSSL Port 2).

A small amount of errors are normal, as long as they are not increasing on subsequent requests.

### (LNQ+REST)

Reset the error count on channel 1 (HSSL Port 1) and channel 2 (HSSL Port 2).

Notes:

- The user should reset the error count first with (LNQ+REST) before requesting link quality.

- The user should perform multiple requests (LNQ?) to see if the error count increase in subsequent request.

- Under normal operating condition, the error count shall be stable and not increasing. Rapidly increasing error count usually indicates cabling issue.